Grupo de Investigaciones Medioambientales

Grupo multidisciplinar que investiga en las áreas de la energía y el medio ambiente con el objetivo final de contribuir a un desarrollo sostenible. Línea de investigación en procesos químicos para la economía circular: Pirolisis de neumáticos fuera de uso, plásticos y biomasa.

Proyecto RECARE:

Reciclaje de residuos que contienen materiales susceptibles de utilizarse como cargas reforzantes de elastómeros, impulsando la economía circular.

La economía circular exige soluciones innovadoras para residuos complejos como la cascarilla de arroz (CA), cuya disposición inadecuada genera importante impactos ambientales. Este proyecto, liderado por el Instituto de Carboquímica (ICB) y Industrias Químicas del Ebro (IQE), busca valorizar la CA mediante pirólisis, un proceso que separa la fracción volátil (celulosa y hemicelulosa) de la lignina, obteniendo una matriz carbonosa con alto contenido de sílice, principal componente de la ceniza.

A diferencia de la combustión tradicional, que convierte el carbono en CO₂ y degrada la sílice amorfa, la pirólisis permite conservar el carbono en forma de biochar y recuperar sílice de alta calidad. El objetivo es emplear estos productos como materias primas secundarias (MPS), específicamente como cargas reforzantes en el sector químico, contribuyendo a la circularidad real.

El proyecto RECARE desarrollará un proceso combinado de pirólisis y lixiviación a escala piloto (TRL-5) para optimizar la extracción de sílice y mejorar las propiedades del biochar.

La candidata/to estará a cargo de todas las campañas experimentales de pirólisis y lixiviación, donde se evaluará el efecto de las diferentes variables del proceso (principalmente temperatura y tiempo de residencia de los volátiles y el sólido) en la calidad del biochar resultante. La interesada/do desarrollará un importante conocimientos teórico y experimental del proceso de pirólisis, y adicionalmente aprenderá técnicas avanzadas de caracterización para todos los productos de la pirólisis. Adicionalmente, por tratarse de un proyecto con la industria, la cadidata/to estará en contacto permanente con la empresa y tendrá la oportunidad de evaluar las MPS obtenidos en formulaciones industriales para determinar su desempeño.

Publicaciones recientes de la línea de investigación:

Pyrolysis-driven chemical recycling: tailoring kinetics to polymer waste complexity

CN Arenas, M Betancur, JD Martínez, MV Navarro

Journal of the Energy Institute 124, 2026, 102348

https://doi.org/10.1016/j.joei.2025.102348

Chemical recycling of EPDM waste through pyrolysis: Exploring the feasibility of a circular economy model

A Sanchís, A Veses, JD Martínez, MS Callén, JM López, T García, R Murillo.

Waste Management 204, 2025, 114928

https://doi.org/10.1016/j.wasman.2025.114928

Pyrolysis of different types and sections of end-of-life tyres: Kinetics and experiments to improve product quality

A Sanchís, MV Navarro, A Veses, JD Martínez, MS Callén, JM López, T García, R Murillo.

Journal of Analytical and Applied Pyrolysis 192, 2025, 107309

https://doi.org/10.1016/j.jaap.2025.107309

On the Exergoeconomics of the Thermochemical Recycling of End-of-Life Tires by Pyrolysis

F Campuzano, AF Agudelo, JD Martínez, WL Roberts

Energy 330, 2025, 136930

https://doi.org/10.1016/j.energy.2025.136930

Waste-based value-added feedstocks from tire pyrolysis oil distillation: defossilization of the petrochemical industry

JD Martínez, A Sanchís, A Veses, A Kapf, JM López, MS Callén, T García, R Murillo.

Green Chemistry 27 (3), 2025, 670-683

https://doi.org/10.1039/D4GC05185H

Recover: Advancements in tire waste management—from collection to disposal and beyond

Chapter 12, In: Rubber Materials, Fundamentals, Sustainability and Applications

JD Martínez, A Veses, T García, R Murillo

Rubber Materials, 359-378

https://doi.org/10.1016/B978-0-443-28989-7.00008-0

Thermochemical recycling of polystyrene waste by pyrolysis using a pilot-scale auger reactor: Process demonstration in a relevant environment

D Royuela, A Veses, JD Martínez, MS Callén, JM López, T García, R Murillo.

Resources, Conservation and Recycling 211, 2024, 107869

https://doi.org/10.1016/j.resconrec.2024.107869

Pyrolysis of polystyrene using low-cost natural catalysts: Production and characterisation of styrene-rich pyro-oils

D Royuela, JD Martínez, MS Callén, JM López, T García, R Murillo, A Veses

Journal of Analytical and Applied Pyrolysis 182, 2024, 106690

https://doi.org/10.1016/j.jaap.2024.106690

Pyrolysis of end-of-life tires: moving from a pilot prototype to a semi-industrial plant using auger technology

A Veses, JD Martínez, A Sanchís, JM López, T García, G García, R Murillo

Energy & Fuels 38 (17), 2024, 17087-17099

https://doi.org/10.1021/acs.energyfuels.4c02748

Pursuing the circularity of wind turbine blades: thermochemical recycling by pyrolysis and recovery of valuable resources

D Royuela, JD Martínez, JM López, MS Callén, T García, R Verdejo, R Murillo, A Veses

Journal of Analytical and Applied Pyrolysis 181, 106657

Design and operation of a packed pilot scale distillation column for tire pyrolysis oil: Towards the recovery of value-added raw materials

JD Martínez, A Sanchís, A Veses, MS Callén, JM López, T García, R Murillo

Fuel 358, 2024, 130266

https://doi.org/10.1016/j.fuel.2023.130266

Pursuing the end-of-life tire circularity: an outlook toward the production of secondary raw materials from tire pyrolysis oil

F Campuzano, JD Martínez, AF Agudelo Santamaría, SM Sarathy, W Roberts.

Energy & Fuels 37 (13), 8836-8866

https://doi.org/10.1021/acs.energyfuels.3c00847

Thermal decomposition characteristics of the tire pyrolysis oil derived from a twin-auger reactor: Study of kinetics and evolved gases

F Campuzano, J Ordoñez, JD Martinez, AF Agudelo, SM Sarathy, W Roberts

Fuel 338, 2023, 127248

https://doi.org/10.1016/j.fuel.2022.127248

On Fractioning the tire pyrolysis oil in a pilot-scale distillation plant under industrially relevant conditions

JD Martínez, A Veses, MS Callén, JM López, T García, R Murillo

Energy & Fuels 37 (4), 2886-2896

https://doi.org/10.1021/acs.energyfuels.2c03850

The role of temperature profile during the pyrolysis of end-of-life-tyres in an industrially relevant conditions auger plant

A Sanchís, A Veses, JD Martínez, JM López, T García, R Murillo

Journal of Environmental Management 317, 2022, 115323

https://doi.org/10.1016/j.jenvman.2022.115323